TABLE _______

ELEVATION REFERENCE MARKS

LUBBOCK COUNTY, TEXAS

CURRENT (1996 MAPS)

| REFERENCE MARK | ELEVATON (FT. NGVD) | DESCRIPTION OF LOCATION | FOUND IN 1998 |
| --- | --- | --- | --- |
| PANEL 0055 |  |  |  |
| RM 1 | 3,106.46 | Railroad spike set in a power pole along a fence line 30 feet south of the centerline of Buffalo Road (State Highway 835) near the southeast corner of the Buffalo Road (State Highway 835) bridge over the North Fork of the Double Mountain Fork of the Brazos River, 400 feet west of city limit sign, 0.23 miles east of the centerline of Loop 289 and 1.6 feet above the natural ground. | NR |
| RM 2 | 3,126.33 | Square in the northwest corner of the headwall of the northbound lane of Loop 289 over the North Fork of the Double Mountain Fork of the Brazos River, bout 0.44 miles north of the Buffalo Road. | NR |
| RM 35 | 3,154.90 | In Lubbock, a cross cut into the west headwall on State Highway 331 as it over passes Loop 289. | No Picture |
| PANEL 0035 |  |  |  |
| RM 3 | 3,157.95 | Railroad spike set in a beam at the northeast corner of the Fort Worth and Denver (Burlington) Railroad trestle over North Fork of the Double Mountain Fork of the Brazos River, about 1,000 feet north of a sewage treatment plant, about 1,500 feet north of 38th Street, 5 feet southeast of the southeast rail, and 5 feet above the top of the fill. | 2-1 2-2 |
| PANEL 0045 |  |  |  |
| RM 41 | 3,225.98 | In Lubbock, a cross cut in the top of curb of the southeast corner of the Intersection of Elgin Avenue and 64th Street. | 2-16 |
| RM 42 | 3,226.50 | In Lubbock, a cross cut in the north end of a headwall on the north side of Loop 289. The headwall is located approximately 1600 feet wet of the intersection of Loop 289 and Indiana Avenue. | NR |
| RM 43 | 3,233.78 | In Lubbock, a bolt on a fire plug on the west side of Quaker Street. The plug is located approximately 60 feet south of the intersection of Quaker Street and 66th Street. | 2-5 |
| RM 44 | 3,255.86 | In Lubbock, a bolt on a fire plug on the east side of Slide Road. The plug is located approximately 20 feet of the intersection of Slide Road and 58th Street. | 2-3 |
| REFERENCE MARK | ELEVATON (FT. NGVD) | DESCRIPTION OF LOCATION | FOUND IN 1998 |
| PANEL 0050 |  |  |  |
| RM 36 | 3,168.39 | In Lubbock, a cross cut in the top of the north curb of East 47th Street at the intersection of East 47th Street and Ute Avenue | 2-10 |
| RM 37 | 3,181.90 | In Lubbock, a cross cut in the top of a drain inlet. The inlet is located on the south side of east 37th Street approximately 30 feet west of the intersection of East 37th Street and Globe Avenue. | 2-9 |
| RM 38 | 3,205.97 | In Lubbock, a cross cut in the top of the sidewalk on the south side of 37th Street. The point is approximately 45 feet east of the intersection of 37th Street and Avenue G. | 2-8 |
| RM 39 | 3,210.66 | In Lubbock, a cross cut in the top of a drain inlet. The inlet is situated on the east side of Avenue T and approximately 40 feet south of the intersection of 42nd Street and Avenue T. | 2-6 |
| RM 40 | 3,211.17 | In Lubbock, a cross cut in the top of curb along the south side of 58th Street. The point is located approximately 15 feet west of the intersection of 58th Street and Avenue T. | 2-7 |
| PANEL 0010 |  |  |  |
| RM 10 | 3,197.16 | Square in the north end of a concrete island on the east side of Amarillo Highway (I-27) at Ash Avenue, 9 feet north of a stop sign, and 2,500 feet north of Ursuline Street | 2-14 |
| RM 11 | 3,219.53 | Bolt on the west side of a fire hydrant, 1 foot above the natural ground, 18 feet west of the centerline of the North Mesa Road (Avenue F) 50 feet south of the centerline of Owens Street | 2-12 |
| RM 12 | 3,200.84 | PK and shiner on the centerline of FM 2641, 100 feet east of the east end of the bridge over Yellow House Draw. | NR |
| RM 13 | 3,207.10 | Railroad spike in a telegraph pole at the Atchison, Topeka and Santa Fe Railway trestle over Yellow House Draw, 38 feet east of the centerline of the railroad track, 0.65 miles north of FM 2641, 2.2 feet above natural ground. | NR 2-15 |
| PANEL 0020 |  |  |  |
| RM 19 | 3,257.07 | In Lubbock, a man-hole circle on the east side of Frankford Road. The manhole is approximately 2,800 feet north of the intersection of Frankford Road and Fourth Street and approximately 2,500 feet south of the intersection of Frankford Road and Erskine Street. | 1-1 |

| REFERENCE MARK | ELEVATON (FT. NGVD) | DESCRIPTION OF LOCATION | FOUND IN 1998 |
| --- | --- | --- | --- |

PANEL 0020 (Continued)

| RM 20 | 3,270.43 | In Lubbock, a railroad spike offset 40.52 feet east of the centerline of Milwaukee Avenue. The point on the centerline is located approximately 1,700 feet north of the intersection of Fourth Street and Milwaukee Avenue and approximately 3,600 feet south of the intersection of Erskine Street and Milwaukee Avenue. | 1-2 |
| --- | --- | --- | --- |
| RM 25 | 3,261.41 | In Lubbock, a cross cut into sidewalk offset 12.95 feet from the east edge of pavement of Frankford Road. The point on Frankford Road is located approximately 3,700 feet north of the intersection of Frankford Road and the Levelland Highway and approximately 1,500 feet south of the intersection of Frankford Road and Fourth Street. | NR |
| RM 33 | 3,264.00 | In Lubbock, a cross cut into the west side of a curb. The curb is located on a park road that intersects the west side of Frankford Road at a point approximately 2,900 feet south of the intersection of Frankford Road and the Levelland Highway. The point on the curb is located along the park road approximately 85 feet southwest from the intersection of the park road and Frankford Road. | NR |
| PANEL 0025 |  |  |  |
| RM 16 | 3,188.03 | “X” cut top center of headwall on north service road of Loop 289 over Yellow House Draw, about 0.7 miles northeast of U.S. Highway 84. (PH 79-140). | NR |
| RM 18 | 3,255.91 | In Lubbock, a railroad spike located 42.71 feet south of the centerline of Erskine Street. The point on Erskine Street is approximately 2,530 feet east of the intersection of Erskine Street and Chicago Avenue and approximately 4,900 feet west of the intersection of Erskine Street and Loop 289. | 1-3 |
| RM 22 | 3,219.96 | In Lubbock, a cross cut in the Hathe curb at the intersection of Indiana Street Duke Street. | NR |
| RM 23 | 3,234.49 | In Lubbock, a railroad spike near a fence line on the west side of Quaker Avenue. The point is approximately 800 feet north of the intersection of Quaker Avenue and Fourth Street and approximately 4,450 feet south of the intersection of Quaker Avenue and Erskine Street. | NR |
| RM 24 | 3,248.96 | In Lubbock, a bolt in a light pole. The pole is located approximately 17 feet south of the edge of pavement of the north lane of Loop 289. The point on Loop 289 is located approximately 6,700 feet southwest of the intersection of Loop 289 and Erskine Street and approximately 6,050 feet northeast of the intersection of Loop 289 and FM 2528. | NR |
| REFERENCE MARK | ELEVATON (FT. NGVD) | DESCRIPTION OF LOCATION | FOUND IN 1998 |
| PANEL 0025 (Continued) |  |  |  |
| RM 30 | 3,210.84 | In Lubbock, a cross cut in the top of curb of the northeast corner of the intersection of Elgin Avenue and 26th Street. | NR |
| RM 31 | 3,221.59 | In Lubbock, a cross cut in the top of the northwest corner of the intersection of 30th Street and Nashville Avenue. | 1-6 |
| RM 32 | 3,248.68 | In Lubbock, a cross cut in the east curb of Slide Road at the intersection of Slide Road and 28th Street. | 2-4 |
| RM 34 | 3,240.76 | In Lubbock, a cross cut into the northeast corner of slab offset approximately 15 feet from the northwest corner of the intersection of Utica Drive and 19th Street. | NR |
| PANEL 0030 |  |  |  |
| RM 4 | 3,180.20 | Square in the center of the curve, on top of the curb, on the northwest corner of east 24th Street at Redbud Avenue. | 2-20 |
| RM 5 | 3,136.76 | Square in the top center of the north culvert headwall of East Broadway over the North Fork of the double Mountain Fork of the Brazos River, 30 feet north of the centerline of East Broadway. | 2-21 |
| RM 6 | 3,177.77 | “X” cut in the centerline of Quirt Avenue, 100’ south of the centerline of Parkway Boulevard (PH 79-78). | NR 2-22 |
| RM 7 | 3,156.84 | PK nail set in the centerline of Park Road #18 about 1,000 feet east of Avenue A, near a maintenance barn, and 135 feet south of the low-water crossing across the North Fork of the Double Mountain Fork of the Brazos River. | NR 2-23 |
| RM 8 | 3,178.15 | Railroad spike set in the north side of a power pole, 25 feet southwest of the centerline of Ash Avenue, 85 feet northwest of the centerline of the Atchison, Topeka and Santa Fe Railway trestle over Blackwater Draw, 4 feet southeast of a corner fence post, and about 2,000 feet south of Harvard Avenue. | 1-10 |
| RM 9 | 3,175.54 | PK nail set in asphalt paving, 50 feet north of the centerline of Erskine Avenue, and 20 feet west of the centerline of Ash Avenue. | 2-24 |
| RM 14 | 3,189.46 | Railroad spike set in a power pole 750 feet south of Erskine Street, 40 feet of their centerline of the Atchison, Topeka and Santa Fe Railway tracks, 23.5 feet north of the Flowline of Yellow House Draw, and 800 feet northwest of Avenue Q. | 1-8 |
| RM 21 | 3,210.73 | In Lubbock, a cross cut in the island at the intersection of Clovis Road and Cornell Street. | NR 2-25 |

| REFERENCE MARK | ELEVATON (FT. NGVD) | DESCRIPTION OF LOCATION | FOUND IN 1998 |
| --- | --- | --- | --- |

PANEL 0030 (Continued)

| RM 27 | 3,181.85 | In Lubbock, a cross cut into the sidewalk of the northeast corner of the intersection of East 28th Street and Ivory Avenue. | NR 2-19 |
| --- | --- | --- | --- |
| RM 28 | 3,189.10 | In Lubbock, a cross cut in the top of curb of the southeast corner of the intersection of 24th Street and Avenue F. | 2-18 |
| RM 29 | 3,201.34 | In Lubbock, a cross cut in the top of curb on the south side of 23rd Street. The point is located approximately 300 feet east of the intersection of 23rd Street and Avenue V and approximately 400 feet west of the intersection of 23rd Street and Avenue U. | 2-17 |