Project Summary

Project name: LubbockHenry.ttp 
Surveyor: S Wood		Linear unit: USFeet	Projection: SPC83-Texas (North Central) NAD83/NAVD88	Geoid: g2003u06

Adjustment Summary

Adjustment type: Minimal constraint;Confidence level: 95  A posteriori standard error of unit weight: 1  Number of adjusted points: 19 
Number of plane control points: 1  Number of height control points: 1   Total number of vectors: 18   Number of used vectors: 18 
Number of rejected vectors: 0

| Used GPS Observations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Solution Type | dn (USft) | de (USft) | du (USft) | Distance (USft) | Horizontal Precision (USft) | Vertical Precision (USft) |
| 2048-2099 | Fixed | -27352.864 | -9857.392 | 46.824 | 29081.578 | 0.015 | 0.026 |
| 2049-2099 | Fixed | -15235.459 | -13893.260 | -68.178 | 20623.879 | 0.009 | 0.014 |
| 2051-2099 | Fixed | 11399.572 | -14059.190 | -47.430 | 18104.293 | 0.008 | 0.012 |
| 2052-2099 | Fixed | 18001.928 | 2407.594 | -137.244 | 18166.988 | 0.010 | 0.016 |
| 2053-2099 | Fixed | -1059.563 | -29487.950 | -73.316 | 29513.933 | 0.014 | 0.031 |
| 2054-2099 | Fixed | -14296.113 | -25777.109 | 117.036 | 29483.016 | 0.012 | 0.030 |
| 2056-2099 | Fixed | -25669.903 | -2994.343 | -4.784 | 25849.930 | 0.013 | 0.023 |
| 2057-2099 | Fixed | -15291.868 | 9330.192 | -113.622 | 17918.056 | 0.012 | 0.024 |
| 2058-2059 | Fixed | 1388.169 | -26091.712 | -15.496 | 26134.667 | 0.015 | 0.023 |
| 2059-2060 | Fixed | -12369.865 | 7746.865 | 11.228 | 14598.847 | 0.008 | 0.012 |
| 2059-2061 | Fixed | -18915.905 | 14664.002 | 25.153 | 23939.731 | 0.020 | 0.031 |
| 2059-2099 | Fixed,Iono Free | -783.237 | 36406.968 | -6.103 | 36423.801 | 0.027 | 0.054 |
| 2062-2099 | Fixed | -9777.029 | -2432.662 | -112.385 | 10078.109 | 0.007 | 0.013 |
| 2063-2099 | Fixed,Iono Free | 24835.577 | -28934.204 | -183.636 | 38140.680 | 0.014 | 0.031 |
| 2064-2099 | Fixed | 6771.061 | 18515.237 | -36.874 | 19719.097 | 0.009 | 0.016 |
| 2098-2099 | Fixed | -9539.387 | -8604.682 | -52.011 | 12849.898 | 0.006 | 0.011 |
| 2099-82048 | Fixed | 22992.416 | 15653.837 | 46.152 | 27821.844 | 0.010 | 0.025 |
| 2099-AE6873 | Fixed | -21969.920 | 7915.585 | 119.379 | 23358.151 | 0.012 | 0.018 |

| GPS Observation Residuals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Res n (USft) | Res e (USft) | Res u (USft) | Status |
| 2048-2099 | 0.001 | 0.000 | 0.000 | Adjusted |
| 2049-2099 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2051-2099 | -0.000 | 0.000 | 0.000 | Adjusted |
| 2052-2099 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2053-2099 | 0.000 | 0.001 | 0.000 | Adjusted |
| 2054-2099 | 0.000 | 0.001 | 0.000 | Adjusted |
| 2056-2099 | 0.001 | 0.000 | 0.000 | Adjusted |
| 2057-2099 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2058-2059 | -0.000 | 0.001 | 0.000 | Adjusted |
| 2059-2060 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2059-2061 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2059-2099 | -0.000 | -0.001 | 0.000 | Adjusted |
| 2062-2099 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2063-2099 | -0.001 | 0.001 | 0.000 | Adjusted |
| 2064-2099 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2098-2099 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2099-82048 | -0.001 | -0.000 | 0.000 | Adjusted |
| 2099-AE6873 | 0.001 | -0.000 | -0.000 | Adjusted |

| Control Points |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Grid Northing (USft) | Grid Easting (USft) | Elevation (USft) | Code |
| 2099 | 7074226.697 | 1176679.540 | 2251.144 |  |
| AE6873 | 7052256.777 | 1184595.125 | 2371.470 |  |

| Adjusted Points |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Grid Northing (USft) | Grid Easting (USft) | Elevation (USft) | Code |
| 2048 | 7101579.562 | 1186536.932 | 2204.652 |  |
| 2049 | 7089462.156 | 1190572.800 | 2319.916 |  |
| 2051 | 7062827.125 | 1190738.730 | 2299.536 |  |
| 2052 | 7056224.769 | 1174271.946 | 2389.051 |  |
| 2053 | 7075286.260 | 1206167.491 | 2325.658 |  |
| 2054 | 7088522.811 | 1202456.650 | 2135.042 |  |
| 2056 | 7099896.600 | 1179673.883 | 2256.118 |  |
| 2057 | 7089518.565 | 1167349.348 | 2364.839 |  |
| 2058 | 7073621.765 | 1166364.284 | 2273.018 |  |
| 2059 | 7075009.934 | 1140272.571 | 2257.051 |  |
| 2060 | 7062640.068 | 1148019.436 | 2268.543 |  |
| 2061 | 7056094.028 | 1154936.573 | 2282.669 |  |
| 2062 | 7084003.726 | 1179112.202 | 2363.916 |  |
| 2063 | 7049391.119 | 1205613.745 | 2436.282 |  |
| 2064 | 7067455.636 | 1158164.303 | 2288.217 |  |
| 2098 | 7083766.085 | 1185284.222 | 2303.695 |  |
| 2099 | 7074226.697 | 1176679.540 | 2251.144 |  |
| 82048 | 7097219.113 | 1192333.377 | 2297.834 |  |
| AE6873 | 7052256.777 | 1184595.125 | 2371.470 |  |

Project Summary
Project name: LubHenryNorth.ttp 
Surveyor: S Wood	Linear unit: USFeet 	Projection: SPC83-Texas (North Central) NAD83/NAVD88	Geoid: g2003u06

Adjustment Summary

Adjustment type: Minimal constraint  Confidence level: 95  A posteriori standard error of unit weight: 1  Number of adjusted points: 3 
Number of plane control points: 1  Number of height control points: 1   Total number of vectors: 2   Number of used vectors: 2 
Number of rejected vectors: 0

| Used GPS Observations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Solution Type | dn (USft) | de (USft) | du (USft) | Distance (USft) | Horizontal Precision (USft) | Vertical Precision (USft) |
| 72048-Kent | Fixed | 7299.177 | 4974.621 | 110.750 | 8835.919 | 0.005 | 0.012 |
| Kent-KentAz | Fixed | 144.055 | -3464.919 | -28.054 | 3468.843 | 0.008 | 0.013 |

| GPS Observation Residuals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Res n (USft) | Res e (USft) | Res u (USft) | Status |
| 72048-Kent | 0.009 | 0.006 | -0.000 | Adjusted |
| Kent-KentAz | 0.000 | -0.004 | 0.000 | Adjusted |

| Control Points |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Grid Northing (USft) | Grid Easting (USft) | Elevation (USft) | Code |
| Kent | 7120614.345 | 1195658.340 | 2412.945 |  |
| KentAz | 7120772.265 | 1192195.548 | 2358.400 |  |

| Adjusted Points |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Grid Northing (USft) | Grid Easting (USft) | Elevation (USft) | Code |
| 72048 | 7113315.177 | 1190683.725 | 2275.760 |  |
| Kent | 7120614.345 | 1195658.340 | 2412.945 |  |
| KentAz | 7120772.265 | 1192195.548 | 2358.400 |  |