Network Adjustment Report

Project : City of Lubbock 2010 Orthophotos

Lake Alan Henry Panels

| User name | R Christopher | Date & Time | 11:17:30 AM 3/1/2010 |
| --- | --- | --- | --- |
| Coordinate System | US State Plane 1983 | Zone | Texas North Central 4202 |
| Project Datum | NAD 1983 (Conus) |  |  |
| Vertical Datum |  | Geoid Model | Geoid03HRA |
| Coordinate Units | US survey feet |  |  |
| Distance Units | US survey feet |  |  |
| Height Units | US survey feet |  |  |

Adjustment Style Settings - 95% Confidence Limits

Residual Tolerances

| To End Iterations | : | 0.000033sft |
| --- | --- | --- |
| Final Convergence Cutoff | : | 0.016404sft |

Covariance Display

| Horizontal |  |  |
| --- | --- | --- |
| Propagated Linear Error [E] | : | U.S. |
| Constant Term [C] | : | 0.00000000sft |
| Scale on Linear Error [S] | : | 1.96 |

| Three-Dimensional |  |  |
| --- | --- | --- |
| Propagated Linear Error [E] | : | U.S. |
| Constant Term [C] | : | 0.00000000sft |
| Scale on Linear Error [S] | : | 1.96 |

Elevation Errors were used in the calculations.

Adjustment Controls

| Compute Correlations for Geoid | : | False |
| --- | --- | --- |

Horizontal and Vertical adjustment performed

Set-up Errors

| GPS |  |  |
| --- | --- | --- |
| Error in Height of Antenna | : | 0.005sft |
| Centering Error | : | 0.003sft |

Back to top

Statistical Summary

Successful Adjustment in 2 iteration(s)

| Network Reference Factor | : | 1.19 |
| --- | --- | --- |
| Chi Square Test (=95%) | : | PASS |
| Degrees of Freedom | : | 15.00 |

GPS Observation Statistics

| Reference Factor | : | 1.19 |
| --- | --- | --- |
| Redundancy Number (r) | : | 15.00 |

Individual GPS Observation Statistics

| Observation ID | Reference Factor | Redundancy Number |
| --- | --- | --- |
| B1 | 2.43 | 0.16 |
| B2 | 0.79 | 1.19 |
| B3 | 0.38 | 0.75 |
| B4 | 0.31 | 1.07 |
| B5 | 0.05 | 0.72 |
| B6 | 1.85 | 1.49 |
| B7 | 0.55 | 1.93 |
| B8 | 0.05 | 1.67 |
| B9 | 2.29 | 2.24 |
| B10 | 0.86 | 2.29 |
| B11 | 0.31 | 1.47 |

Geoid Model Statistics

| Reference Factor | : | 1.00 |
| --- | --- | --- |
| Redundancy Number (r) | : | 0.00 |

Weighting Strategies

GPS Observations
User-defined Scalar Applied to All Observations

| Scalar | : | 9.33 |
| --- | --- | --- |

Geoid Observations
Alternative Scalar Applied to All Observations

| Scalar | : | 1.00 |
| --- | --- | --- |

Adjusted Coordinates

Adjustment performed in NAD 1983 (Conus)

| Number of Points | : | 7 |
| --- | --- | --- |
| Number of Constrained Points | : | 1 |
| Horizontal plus Height and Elevation | : | 1 |

Adjusted Grid Coordinates

Errors are reported using 1.96.

| Point Name | Northing | N error | Easting | E error | Elevation | e error | Fix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2098 | 7083766.085sft | 0.000sft | 1185284.222sft | 0.000sft | 2303.532sft | 0.000sft | N E h e |
| 82048 | 7094301.571sft | 0.045sft | 1190631.496sft | 0.038sft | 2321.676sft | 2.469sft |  |
| 2062 | 7084003.704sft | 0.079sft | 1179112.134sft | 0.064sft | 2363.585sft | 2.477sft |  |
| 2049 | 7089462.164sft | 0.082sft | 1190572.817sft | 0.068sft | 2319.599sft | 2.483sft |  |
| 2099 | 7074217.352sft | 0.088sft | 1176682.525sft | 0.069sft | 2253.498sft | 2.485sft |  |
| 2061 | 7062823.368sft | 0.116sft | 1190741.416sft | 0.086sft | 2299.150sft | 2.486sft |  |
| TXLU | 7257752.795sft | 0.073sft | 950076.199sft | 0.058sft | 3221.800sft | 2.477sft |  |

Adjusted Geodetic Coordinates

Errors are reported using 1.96.

| Point Name | Latitude | N error | Longitude | E error | Height | h error | Fix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2098 | 33°04'31.97414"N | 0.000sft | 101°03'26.07169"W | 0.000sft | 2219.080sft | 0.000sft | Lat Long h e |
| 82048 | 33°06'17.47635"N | 0.045sft | 101°02'26.23764"W | 0.038sft | 2237.298sft | 0.106sft |  |
| 2062 | 33°04'32.83232"N | 0.079sft | 101°04'38.65914"W | 0.064sft | 2279.349sft | 0.227sft |  |
| 2049 | 33°05'29.59018"N | 0.082sft | 101°02'25.55157"W | 0.068sft | 2235.156sft | 0.285sft |  |
| 2099 | 33°02'55.43400"N | 0.088sft | 101°05'04.37641"W | 0.069sft | 2169.169sft | 0.301sft |  |
| 2061 | 33°01'06.11357"N | 0.116sft | 101°02'16.00638"W | 0.086sft | 2214.340sft | 0.310sft |  |
| TXLU | 33°32'07.49023"N | 0.073sft | 101°50'34.16386"W | 0.058sft | 3140.359sft | 0.229sft |  |

Coordinate Deltas

| Point Name | Northing | Easting | Elevation | Height | Geoid Separation |
| --- | --- | --- | --- | --- | --- |
| 2098 | 0.000sft | 0.000sft | 0.000sft | 0.000sft | 0.000sft |
| 82048 | 0.004sft | -0.001sft | 0.080sft | 0.080sft | 0.000sft |
| 2062 | -0.002sft | 0.000sft | -0.028sft | -0.028sft | 0.000sft |
| 2049 | -0.001sft | -0.003sft | 0.056sft | 0.056sft | 0.000sft |
| 2099 | -0.009sft | -0.002sft | -0.081sft | -0.081sft | 0.000sft |
| 2061 | -0.007sft | 0.002sft | -0.087sft | -0.087sft | 0.000sft |
| TXLU | -0.042sft | -0.066sft | 0.000sft | 0.000sft | 0.000sft |

Control Coordinate Comparisons

Values shown are control coord minus adjusted coord.

| Point Name | Northing | Easting | Elevation | Height |
| --- | --- | --- | --- | --- |
| 2098 | N/A | N/A | N/A | N/A |
| TXLU | 0.042sft | 0.066sft | N/A | 0.000sft |

Adjusted Observations

Adjustment performed in NAD 1983 (Conus)

GPS Observations

| Number of Observations | : | 11 |
| --- | --- | --- |
| Number of Outliers | : | 0 |

Observation Adjustment (Critical Tau = 2.83).  Any outliers are in red.

| Obs. ID | From Pt. | To Pt. |  | Observation | A-posteriori Error (1.96) | Residual | Stand. Residual |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | 2098 | 82048 | Az. | 25°30'55.1125" | 0°00'00.6389" | 0°00'00.0561" | 0.76 |
|  |  |  | Ht. | 18.218sft | 0.106sft | 0.028sft | 2.38 |
|  |  |  | Dist. | 11816.315sft | 0.046sft | 0.015sft | 2.30 |
| B6 | TXLU | 2098 | Az. | 124°39'56.8594" | 0°00'00.0487" | -0°00'00.0187" | -0.80 |
|  |  |  | Ht. | -921.279sft | 0.229sft | 0.329sft | 2.37 |
|  |  |  | Dist. | 292598.463sft | 0.063sft | -0.004sft | -0.13 |
| B9 | 82048 | TXLU | Az. | 302°48'38.2467" | 0°00'00.0553" | 0°00'00.0952" | 1.93 |
|  |  |  | Ht. | 903.061sft | 0.239sft | 0.427sft | 2.29 |
|  |  |  | Dist. | 290865.004sft | 0.069sft | -0.072sft | -0.99 |
| B10 | 2062 | TXLU | Az. | 305°46'46.6579" | 0°00'00.0460" | -0°00'00.0473" | -0.98 |
|  |  |  | Ht. | 861.010sft | 0.237sft | 0.110sft | 0.59 |
|  |  |  | Dist. | 287515.563sft | 0.063sft | 0.012sft | 0.19 |
| B2 | 2098 | 2062 | Az. | 270°48'36.1201" | 0°00'02.6412" | -0°00'01.0104" | -0.76 |
|  |  |  | Ht. | 60.269sft | 0.227sft | -0.011sft | -0.12 |
|  |  |  | Dist. | 6177.447sft | 0.063sft | 0.022sft | 0.79 |
| B7 | TXLU | 2049 | Az. | 123°09'27.9703" | 0°00'00.0401" | 0°00'00.0016" | 0.07 |
|  |  |  | Ht. | -905.203sft | 0.241sft | 0.144sft | 0.64 |
|  |  |  | Dist. | 293564.446sft | 0.055sft | 0.001sft | 0.02 |
| B3 | 2062 | 2049 | Az. | 63°07'36.8352" | 0°00'00.8732" | 0°00'00.0179" | 0.08 |
|  |  |  | Ht. | -44.193sft | 0.245sft | -0.043sft | -0.46 |
|  |  |  | Dist. | 12695.789sft | 0.052sft | 0.003sft | 0.17 |
| B11 | TXLU | 2099 | Az. | 127°10'48.4684" | 0°00'00.0359" | 0°00'00.0022" | 0.17 |
|  |  |  | Ht. | -971.190sft | 0.236sft | -0.057sft | -0.23 |
|  |  |  | Dist. | 291642.496sft | 0.048sft | 0.006sft | 0.37 |
| B4 | 2049 | 2099 | Az. | 220°57'09.5387" | 0°00'00.6015" | 0°00'00.1046" | 0.36 |
|  |  |  | Ht. | -65.987sft | 0.209sft | -0.003sft | -0.04 |
|  |  |  | Dist. | 20626.508sft | 0.059sft | -0.002sft | -0.08 |
| B8 | TXLU | 2061 | Az. | 127°10'52.0358" | 0°00'00.0585" | -0°00'00.0011" | -0.03 |
|  |  |  | Ht. | -926.019sft | 0.236sft | -0.008sft | -0.08 |
|  |  |  | Dist. | 309741.078sft | 0.073sft | -0.001sft | -0.02 |
| B5 | 2099 | 2061 | Az. | 127°36'47.9961" | 0°00'00.9310" | 0°00'00.0077" | 0.03 |
|  |  |  | Ht. | 45.171sft | 0.184sft | 0.003sft | 0.08 |
|  |  |  | Dist. | 18098.583sft | 0.066sft | 0.000sft | 0.02 |

Geoid Observations

| Number of Observations | : | 6 |
| --- | --- | --- |
| Number of Outliers | : | 0 |

Observation Adjustment (Critical Tau = 2.83).  Any outliers are in red.

| Observation ID | Point Name | Separation | A-posteriori Error (1.96) | Residual | Standardized Residual |
| --- | --- | --- | --- | --- | --- |
| G7 | TXLU | -81.441sft | 2.466sft | 0.000sft | 0.00 |
| G1 | 2098 | ? | ? | ? | ? |
| G2 | 82048 | -84.378sft | 2.466sft | 0.000sft | 0.00 |
| G3 | 2062 | -84.236sft | 2.466sft | 0.000sft | 0.00 |
| G4 | 2049 | -84.443sft | 2.466sft | 0.000sft | 0.00 |
| G5 | 2099 | -84.329sft | 2.466sft | 0.000sft | 0.00 |
| G6 | 2061 | -84.810sft | 2.466sft | 0.000sft | 0.00 |

Histograms of Standardized Residuals

Point Error Ellipses

| 82048 | 2062 | 2049 |
| --- | --- | --- |
| Tick Size: 0.1000sft Horizontal Bivariate Scalar: 2.45 Vertical Univariate Scalar: 1.96 |  |  |
| 2099 | 2061 | TXLU |
| Tick Size: 0.1000sft Horizontal Bivariate Scalar: 2.45 Vertical Univariate Scalar: 1.96 |  |  |

Covariant Terms

Adjustment performed in NAD 1983 (Conus)

| From Point | To Point |  | Components | A-posteriori Error (1.96) | Horiz. Precision (Ratio) | 3D Precision (Ratio) |
| --- | --- | --- | --- | --- | --- | --- |
| 2098 | 82048 | Az. | 25°30'55.1125" | 0°00'00.6389" | 1:255438 | 1:255438 |
|  |  | Ht. | 18.218sft | 0.106sft |  |  |
|  |  | Elev. | 18.144sft | 2.469sft |  |  |
|  |  | Dist. | 11816.315sft | 0.046sft |  |  |
| 2098 | 2062 | Az. | 270°48'36.1201" | 0°00'02.6412" | 1:97284 | 1:97284 |
|  |  | Ht. | 60.269sft | 0.227sft |  |  |
|  |  | Elev. | 60.053sft | 2.477sft |  |  |
|  |  | Dist. | 6177.447sft | 0.063sft |  |  |
| 2098 | TXLU | Az. | 305°05'49.8022" | 0°00'00.0485" | 1:4671455 | 1:4671455 |
|  |  | Ht. | 921.279sft | 0.229sft |  |  |
|  |  | Elev. | 918.268sft | 2.477sft |  |  |
|  |  | Dist. | 292598.463sft | 0.063sft |  |  |
| 82048 | TXLU | Az. | 302°48'38.2467" | 0°00'00.0553" | 1:4209079 | 1:4209079 |
|  |  | Ht. | 903.061sft | 0.239sft |  |  |
|  |  | Elev. | 900.123sft | 3.497sft |  |  |
|  |  | Dist. | 290865.004sft | 0.069sft |  |  |
| 2062 | 2049 | Az. | 63°07'36.8352" | 0°00'00.8732" | 1:243003 | 1:243003 |
|  |  | Ht. | -44.193sft | 0.245sft |  |  |
|  |  | Elev. | -43.986sft | 3.507sft |  |  |
|  |  | Dist. | 12695.789sft | 0.052sft |  |  |
| 2062 | TXLU | Az. | 305°46'46.6579" | 0°00'00.0460" | 1:4529240 | 1:4529240 |
|  |  | Ht. | 861.010sft | 0.237sft |  |  |
|  |  | Elev. | 858.215sft | 3.503sft |  |  |
|  |  | Dist. | 287515.563sft | 0.063sft |  |  |
| 2049 | 2099 | Az. | 220°57'09.5387" | 0°00'00.6015" | 1:352299 | 1:352299 |
|  |  | Ht. | -65.987sft | 0.209sft |  |  |
|  |  | Elev. | -66.101sft | 3.512sft |  |  |
|  |  | Dist. | 20626.508sft | 0.059sft |  |  |
| 2049 | TXLU | Az. | 303°35'54.4825" | 0°00'00.0400" | 1:5317371 | 1:5317371 |
|  |  | Ht. | 905.203sft | 0.241sft |  |  |
|  |  | Elev. | 902.201sft | 3.507sft |  |  |
|  |  | Dist. | 293564.446sft | 0.055sft |  |  |
| 2099 | 2061 | Az. | 127°36'47.9961" | 0°00'00.9310" | 1:272181 | 1:272181 |
|  |  | Ht. | 45.171sft | 0.184sft |  |  |
|  |  | Elev. | 45.652sft | 3.515sft |  |  |
|  |  | Dist. | 18098.583sft | 0.066sft |  |  |
| 2099 | TXLU | Az. | 307°35'46.8971" | 0°00'00.0357" | 1:6037734 | 1:6037734 |
|  |  | Ht. | 971.190sft | 0.236sft |  |  |
|  |  | Elev. | 968.302sft | 3.508sft |  |  |
|  |  | Dist. | 291642.496sft | 0.048sft |  |  |
| 2061 | TXLU | Az. | 307°37'22.2477" | 0°00'00.0583" | 1:4226056 | 1:4226056 |
|  |  | Ht. | 926.019sft | 0.236sft |  |  |
|  |  | Elev. | 922.650sft | 3.509sft |  |  |
|  |  | Dist. | 309741.078sft | 0.073sft |  |  |