Project Summary

Project name: LubbockBase.ttp 
Surveyor: S Wood	Linear unit: USFeet Projection: SPC83-Texas (North Central) NAD83/NAVD88 Geoid: g2003u06

Adjustment Summary

Adjustment type: Constraint Confidence level: 95 A posteriori standard error of unit weight: 2.194509

Number of adjusted points: 83 Number of plane control points: 2 Number of height control points: 5

Total number of vectors: 85 Number of used vectors: 82

| Used GPS Observations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Solution Type | dn (USft) | de (USft) | du (USft) | Distance (USft) | Horizontal Precision (USft) | Vertical Precision (USft) |
| 1262-2006 | Fixed | 4922.042 | 8668.663 | -40.192 | 9970.901 | 0.020 | 0.051 |
| 1300-2006 | Fixed | 536.081 | -17814.572 | 12.878 | 17826.628 | 0.012 | 0.017 |
| 1495-2025 | Fixed | -5768.839 | 10985.667 | -29.321 | 12411.198 | 0.010 | 0.014 |
| 2001-2096 | Fixed | -5231.689 | -131.701 | 3.311 | 5234.547 | 0.023 | 0.032 |
| 2002-2006 | Fixed | 322.301 | -12737.563 | 6.781 | 12744.495 | 0.011 | 0.014 |
| 2003-2006 | Fixed | 158.778 | -7271.675 | 4.636 | 7275.039 | 0.007 | 0.011 |
| 2004-2006 | Fixed | -83.438 | 3141.785 | -10.221 | 3143.614 | 0.005 | 0.010 |
| 2005-2006 | Fixed | -498.331 | 13728.633 | -43.926 | 13740.837 | 0.007 | 0.014 |
| 2006-2096 | Fixed | -4645.216 | -19270.912 | 64.566 | 19827.472 | 0.026 | 0.039 |
| 2006-807 | Fixed,Iono Free | -3499.638 | 38766.000 | 3.811 | 38932.388 | 0.023 | 0.047 |
| 2006-81154 | Fixed | -31347.841 | -9259.352 | 36.951 | 32694.375 | 0.048 | 0.065 |
| 2006-81218 | Fixed | -15303.306 | -14299.081 | 64.806 | 20949.004 | 0.017 | 0.023 |
| 2006-81260 | Fixed | -4768.656 | -13943.423 | 55.197 | 14739.765 | 0.016 | 0.031 |
| 2006-81293 | Fixed | -5591.833 | 12444.544 | -21.886 | 13646.227 | 0.016 | 0.021 |
| 2006-81294 | Fixed | -5561.790 | 6711.165 | -7.344 | 8718.231 | 0.017 | 0.028 |
| 2006-81296 | Fixed | -5601.248 | 1863.189 | 10.862 | 5904.349 | 0.008 | 0.019 |
| 2006-81298 | Fixed | -5088.828 | -3308.811 | 16.036 | 6071.353 | 0.009 | 0.018 |
| 2007-2009 | Fixed | -31586.271 | -788.106 | -19.592 | 31603.708 | 0.020 | 0.030 |
| 2008-2020 | Fixed,Iono Free | -6313.880 | 33887.737 | -46.119 | 34479.774 | 0.030 | 0.044 |
| 2009-2096 | Fixed,Iono Free | 36373.905 | 16859.162 | 0.899 | 40100.620 | 0.054 | 0.071 |
| 2009-4001 | Fixed,Iono Free | 30349.184 | -19489.256 | 61.239 | 36076.818 | 0.023 | 0.037 |
| 2009-4002 | Fixed | -12121.981 | -20937.786 | 44.566 | 24199.750 | 0.027 | 0.035 |
| 2009-4008 | Fixed | 1743.830 | -20274.900 | 50.488 | 20354.847 | 0.013 | 0.016 |
| 2009-4009 | Fixed | 18636.389 | -19459.124 | 71.107 | 26950.556 | 0.011 | 0.022 |
| 2009-4010 | Fixed | 12430.371 | -11374.827 | 34.373 | 16853.517 | 0.011 | 0.020 |
| 2009-4011 | Fixed | -3747.821 | -20449.597 | 42.276 | 20795.395 | 0.009 | 0.017 |
| 2010-81107 | Fixed | 179.414 | -5416.034 | 12.093 | 5420.376 | 0.023 | 0.038 |
| 2011-81107 | Fixed | 378.521 | -10589.545 | 23.704 | 10598.985 | 0.018 | 0.027 |
| 2012-2020 | Fixed,Iono Free | -1116.471 | 34058.058 | -41.508 | 34085.133 | 0.032 | 0.051 |
| 2013-81107 | Fixed | 5631.749 | -10298.698 | 29.268 | 11740.947 | 0.019 | 0.037 |
| 2014-81107 | Fixed | 5450.233 | -5183.378 | 16.701 | 7523.376 | 0.015 | 0.025 |
| 2015-81107 | Fixed | 5336.438 | 286.226 | 5.187 | 5345.459 | 0.022 | 0.047 |
| 2016-81107 | Fixed | 4780.813 | 16091.036 | -41.639 | 16790.526 | 0.029 | 0.039 |
| 2017-2020 | Fixed | -261.973 | 7650.743 | 2.037 | 7657.187 | 0.012 | 0.019 |
| 2018-2020 | Fixed | -766.511 | 23551.217 | -29.805 | 23569.755 | 0.015 | 0.021 |
| 2019-2020 | Fixed | -497.759 | 15930.579 | -31.929 | 15942.478 | 0.009 | 0.016 |
| 2020-4007 | Fixed,Iono Free | 16961.092 | -33551.852 | 44.471 | 37604.875 | 0.031 | 0.060 |
| 2020-81045 | Fixed | 5725.452 | -12725.123 | 18.632 | 13957.417 | 0.028 | 0.047 |
| 2020-81067 | Fixed | 5257.741 | -2169.560 | 9.446 | 5689.243 | 0.014 | 0.019 |
| 2020-81068 | Fixed | 5105.514 | 3230.321 | 1.251 | 6043.171 | 0.024 | 0.039 |
| 2020-81107 | Fixed | 4377.724 | 29505.617 | -73.813 | 29836.257 | 0.019 | 0.028 |
| 2020-81183 | Fixed | 21171.861 | 4035.419 | 1.491 | 21558.466 | 0.014 | 0.025 |
| 2020-83098 | Fixed | 6162.494 | -28544.052 | 46.412 | 29209.219 | 0.022 | 0.035 |
| 2021-81107 | Fixed | 5057.726 | 7405.050 | -9.981 | 8969.728 | 0.021 | 0.029 |
| 2022-2025 | Fixed | 4602.884 | 10912.671 | 1.175 | 11846.496 | 0.006 | 0.010 |
| 2023-2025 | Fixed | -10615.694 | 272.958 | -11.239 | 10621.702 | 0.009 | 0.022 |
| 2024-2025 | Fixed | -4325.053 | -448.233 | -5.154 | 4349.246 | 0.005 | 0.008 |
| 2025-2026 | Fixed | -5392.830 | 760.791 | -45.192 | 5447.707 | 0.007 | 0.011 |
| 2025-2027 | Fixed | -10286.398 | 24.776 | -3.576 | 10288.884 | 0.007 | 0.012 |
| 2025-2028 | Fixed | 3962.239 | 11579.691 | -10.653 | 12241.695 | 0.026 | 0.048 |
| 2025-2029 | Fixed | -1453.375 | 16722.249 | -22.328 | 16789.260 | 0.026 | 0.060 |
| 2025-2030 | Fixed | -6492.084 | 16468.926 | -26.292 | 17706.545 | 0.029 | 0.049 |
| 2025-2031 | Fixed | -12073.087 | 16434.091 | -40.297 | 20397.010 | 0.012 | 0.021 |
| 2025-808 | Fixed | 23657.442 | -11991.071 | 46.753 | 26529.037 | 0.014 | 0.025 |
| 2025-81493 | Fixed | 11164.617 | -10771.270 | 45.251 | 15517.222 | 0.009 | 0.013 |
| 2025-81496 | Fixed | 556.578 | -11337.827 | 21.365 | 11354.185 | 0.006 | 0.009 |
| 2025-81499 | Fixed | -10007.541 | -9925.450 | 5.197 | 14098.236 | 0.010 | 0.016 |
| 2025-81501 | Fixed | -15211.070 | -11092.071 | 3.332 | 18830.311 | 0.021 | 0.033 |
| 2025-82031 | Fixed | -9166.985 | 15602.964 | -188.300 | 18101.781 | 0.010 | 0.017 |
| 2032-2035 | Fixed | -176.182 | 6632.015 | 1.794 | 6635.780 | 0.006 | 0.010 |
| 2033-2035 | Fixed | 410.616 | -14507.209 | 4.736 | 14516.136 | 0.009 | 0.015 |
| 2034-2035 | Fixed | 254.799 | -8095.384 | 4.183 | 8101.133 | 0.012 | 0.016 |
| 2035-4003 | Fixed | 10742.310 | -8447.198 | 17.889 | 13668.658 | 0.007 | 0.011 |
| 2035-4004 | Fixed,Iono Free | 9727.799 | 32111.992 | -23.746 | 33560.193 | 0.014 | 0.024 |
| 2035-4006 | Fixed | 14915.759 | 14899.652 | 15.083 | 21087.142 | 0.010 | 0.016 |
| 2035-807 | Fixed | -23695.287 | 8548.693 | -36.668 | 25195.801 | 0.011 | 0.016 |
| 2035-808 | Fixed,Iono Free | -35688.740 | 22671.697 | -98.743 | 42290.609 | 0.023 | 0.037 |
| 2035-81383 | Fixed | -5271.729 | -6692.780 | -12.473 | 8521.502 | 0.007 | 0.011 |
| 2035-81387 | Fixed | -5270.773 | -1616.676 | -23.094 | 5514.380 | 0.007 | 0.011 |
| 2035-81388 | Fixed | -5518.739 | 3441.602 | -6.918 | 6505.343 | 0.007 | 0.012 |
| 2035-81389 | Fixed | -5670.951 | 9008.224 | -9.624 | 10646.920 | 0.012 | 0.019 |
| 2035-81390 | Fixed | -5639.883 | 14197.698 | -11.645 | 15280.181 | 0.013 | 0.015 |
| 2035-81411 | Fixed | -10899.635 | 24785.145 | -54.661 | 27081.834 | 0.015 | 0.019 |
| 4005-808 | Fixed | -12269.001 | -11711.722 | 5.931 | 16965.331 | 0.051 | 0.062 |
| 81107-81109 | Fixed | 5241.101 | 195.743 | 3.466 | 5246.071 | 0.014 | 0.020 |
| 81107-81113 | Fixed | 5074.557 | 5402.225 | -18.804 | 7413.700 | 0.019 | 0.029 |
| 81107-81115 | Fixed | 4935.471 | 10703.448 | -32.954 | 11789.529 | 0.015 | 0.020 |
| 81109-9099 | Fixed | 26885.064 | 377.219 | 9.337 | 26894.296 | 0.017 | 0.034 |
| 81283-9099 | Fixed | -2615.245 | 15254.075 | -22.992 | 15480.357 | 0.010 | 0.017 |
| 81448-9099 | Fixed | 13686.427 | -18.759 | -7.363 | 13689.764 | 0.007 | 0.011 |
| 81475-9099 | Fixed | 998.731 | -15786.066 | 50.406 | 15821.472 | 0.011 | 0.019 |
| 81496-9099 | Fixed | 1270.754 | -26645.797 | 49.175 | 26682.475 | 0.021 | 0.031 |

| GPS Observation Residuals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Res n (USft) | Res e (USft) | Res u (USft) | Status |
| 1262-2006 | 0.000 | -0.000 | -0.000 | Adjusted |
| 1300-2006 | -0.000 | 0.000 | -0.000 | Adjusted |
| 1495-2025 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2001-2096 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2002-2006 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2003-2006 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2004-2006 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2005-2006 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2006-2096 | -0.000 | 0.000 | 0.000 | Adjusted |
| 2006-807 | 0.000 | -0.000 | -0.085 | Adjusted |
| 2006-81154 | -0.000 | 0.000 | 0.000 | Adjusted |
| 2006-81218 | -0.000 | 0.000 | -0.000 | Adjusted |
| 2006-81260 | -0.000 | 0.000 | -0.000 | Adjusted |
| 2006-81293 | -0.000 | -0.000 | -0.000 | Adjusted |
| 2006-81294 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2006-81296 | -0.000 | 0.000 | 0.000 | Adjusted |
| 2006-81298 | -0.000 | 0.000 | -0.000 | Adjusted |
| 2007-2009 | -0.000 | 0.000 | -0.000 | Adjusted |
| 2008-2020 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2009-2096 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2009-4001 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2009-4002 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2009-4008 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2009-4009 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2009-4010 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2009-4011 | -0.000 | -0.000 | -0.000 | Adjusted |
| 2010-81107 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2011-81107 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2012-2020 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2013-81107 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2014-81107 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2015-81107 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2016-81107 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2017-2020 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2018-2020 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2019-2020 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2020-4007 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2020-81045 | -0.000 | -0.000 | -0.000 | Adjusted |
| 2020-81067 | -0.000 | -0.000 | -0.000 | Adjusted |
| 2020-81068 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2020-81107 | 0.000 | 0.000 | 0.016 | Adjusted |
| 2020-81183 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2020-83098 | -0.000 | -0.000 | -0.000 | Adjusted |
| 2021-81107 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2022-2025 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2023-2025 | -0.000 | -0.000 | -0.000 | Adjusted |
| 2024-2025 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2025-2026 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2025-2027 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2025-2028 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2025-2029 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2025-2030 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2025-2031 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2025-808 | 0.014 | -0.018 | -0.196 | Auto-Rejected |
| 2025-81493 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2025-81496 | -0.002 | 0.004 | 0.002 | Adjusted |
| 2025-81499 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2025-81501 | -0.000 | 0.000 | -0.000 | Adjusted |
| 2025-82031 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2032-2035 | 0.000 | -0.000 | 0.000 | Adjusted |
| 2033-2035 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2034-2035 | -0.000 | 0.000 | -0.000 | Adjusted |
| 2035-4003 | 0.000 | -0.000 | -0.000 | Adjusted |
| 2035-4004 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2035-4006 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2035-807 | -0.000 | 0.000 | 0.012 | Adjusted |
| 2035-808 | 0.000 | -0.000 | -0.054 | Adjusted |
| 2035-81383 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2035-81387 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2035-81388 | 0.000 | 0.000 | 0.000 | Adjusted |
| 2035-81389 | 0.000 | 0.000 | -0.000 | Adjusted |
| 2035-81390 | -0.000 | -0.000 | 0.000 | Adjusted |
| 2035-81411 | 0.000 | 0.000 | 0.000 | Adjusted |
| 4005-808 | 0.000 | 0.000 | 0.000 | Adjusted |
| 808-81493 | -0.007 | 0.110 | 0.195 | Disabled from Adjustment |
| 81107-81109 | 0.000 | 0.000 | 0.009 | Adjusted |
| 81107-81113 | 0.000 | 0.000 | -0.000 | Adjusted |
| 81107-81115 | 0.000 | 0.000 | 0.000 | Adjusted |
| 81109-9099 | 0.000 | -0.000 | 0.024 | Adjusted |
| 81283-9099 | 0.000 | 0.000 | 0.000 | Adjusted |
| 81448-9099 | 0.000 | -0.000 | 0.000 | Adjusted |
| 81475-9099 | 0.000 | 0.000 | 0.000 | Adjusted |
| 81496-9099 | -0.024 | 0.020 | 0.015 | Adjusted |

| Control Points |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Grid Northing (USft) | Grid Easting (USft) | Elevation (USft) | Code |
| 2006 | 7300541.734 | 919785.981 | 3247.452 |  |
| 2020 | 7226714.684 | 916603.812 | 3272.669 |  |
| 2025 | 7261391.215 | 984666.038 | 3142.876 |  |
| 808 | 7285048.643 | 972674.985 | 3190.030 |  |
| 9099 | 7263218.573 | 946682.391 | 3212.771 |  |

| Adjusted Points |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Grid Northing (USft) | Grid Easting (USft) | Elevation (USft) | Code |
| 1262 | 7295619.692 | 911117.318 | 3287.370 |  |
| 1300 | 7300005.653 | 937600.553 | 3234.917 |  |
| 1495 | 7267160.054 | 973680.371 | 3172.125 |  |
| 2001 | 7301128.207 | 900646.770 | 3308.285 |  |
| 2002 | 7300219.434 | 932523.544 | 3240.921 |  |
| 2003 | 7300382.956 | 927057.656 | 3242.962 |  |
| 2004 | 7300625.173 | 916644.196 | 3257.608 |  |
| 2005 | 7301040.066 | 906057.348 | 3291.081 |  |
| 2006 | 7300541.734 | 919785.981 | 3247.452 |  |
| 2007 | 7291108.885 | 884444.013 | 3329.738 |  |
| 2008 | 7233028.564 | 882716.075 | 3318.191 |  |
| 2009 | 7259522.613 | 883655.907 | 3309.603 |  |
| 2010 | 7230912.994 | 951525.463 | 3187.513 |  |
| 2011 | 7230713.887 | 956698.974 | 3176.000 |  |
| 2012 | 7227831.155 | 882545.754 | 3313.517 |  |
| 2013 | 7225460.659 | 956408.127 | 3170.370 |  |
| 2014 | 7225642.175 | 951292.807 | 3182.839 |  |
| 2015 | 7225755.970 | 945823.203 | 3194.244 |  |
| 2016 | 7226311.595 | 930018.393 | 3240.750 |  |
| 2017 | 7226976.657 | 908953.069 | 3270.482 |  |
| 2018 | 7227481.195 | 893052.595 | 3302.019 |  |
| 2019 | 7227212.443 | 900673.233 | 3304.287 |  |
| 2020 | 7226714.684 | 916603.812 | 3272.669 |  |
| 2021 | 7226034.682 | 938704.379 | 3209.269 |  |
| 2022 | 7256788.331 | 973753.368 | 3141.462 |  |
| 2023 | 7272006.909 | 984393.080 | 3154.277 |  |
| 2024 | 7265716.268 | 985114.271 | 3148.103 |  |
| 2025 | 7261391.215 | 984666.038 | 3142.876 |  |
| 2026 | 7255998.385 | 985426.829 | 3097.612 |  |
| 2027 | 7251104.817 | 984690.815 | 3139.144 |  |
| 2028 | 7265353.454 | 996245.730 | 3132.434 |  |
| 2029 | 7259937.840 | 1001388.288 | 3120.746 |  |
| 2030 | 7254899.131 | 1001134.965 | 3116.713 |  |
| 2031 | 7249318.128 | 1001100.129 | 3102.638 |  |
| 2032 | 7320913.565 | 943371.273 | 3287.051 |  |
| 2033 | 7320326.768 | 964510.496 | 3284.460 |  |
| 2034 | 7320482.584 | 958098.671 | 3284.913 |  |
| 2035 | 7320737.383 | 950003.288 | 3288.963 |  |
| 2096 | 7295896.518 | 900515.069 | 3311.504 |  |
| 4001 | 7289871.797 | 864166.651 | 3370.901 |  |
| 4002 | 7247400.633 | 862718.121 | 3353.552 |  |
| 4003 | 7331479.693 | 941556.089 | 3306.883 |  |
| 4004 | 7330465.182 | 982115.280 | 3265.900 |  |
| 4005 | 7297317.644 | 984386.707 | 3184.475 |  |
| 4006 | 7335653.142 | 964902.940 | 3304.557 |  |
| 4007 | 7243675.776 | 883051.960 | 3316.676 |  |
| 4008 | 7261266.444 | 863381.007 | 3359.688 |  |
| 4009 | 7278159.003 | 864196.783 | 3380.585 |  |
| 4010 | 7271952.984 | 872281.080 | 3343.930 |  |
| 4011 | 7255774.793 | 863206.310 | 3351.393 |  |
| 807 | 7297042.096 | 958551.981 | 3252.011 |  |
| 808 | 7285048.643 | 972674.985 | 3190.030 |  |
| 81045 | 7232440.136 | 903878.689 | 3291.115 |  |
| 81067 | 7231972.425 | 914434.252 | 3282.133 |  |
| 81068 | 7231820.198 | 919834.133 | 3274.045 |  |
| 81107 | 7231092.408 | 946109.429 | 3199.499 |  |
| 81109 | 7236333.509 | 946305.172 | 3203.027 |  |
| 81113 | 7236166.965 | 951511.654 | 3180.868 |  |
| 81115 | 7236027.879 | 956812.877 | 3166.819 |  |
| 81154 | 7269193.893 | 910526.629 | 3283.631 |  |
| 81183 | 7247886.545 | 920639.231 | 3274.526 |  |
| 81218 | 7285238.428 | 905486.900 | 3311.669 |  |
| 81260 | 7295773.078 | 905842.557 | 3302.256 |  |
| 81283 | 7265833.818 | 931428.316 | 3235.503 |  |
| 81293 | 7294949.901 | 932230.525 | 3225.719 |  |
| 81294 | 7294979.944 | 926497.145 | 3240.150 |  |
| 81296 | 7294940.487 | 921649.170 | 3258.257 |  |
| 81298 | 7295452.907 | 916477.169 | 3263.330 |  |
| 81383 | 7315465.654 | 943310.508 | 3276.276 |  |
| 81387 | 7315466.610 | 948386.612 | 3265.747 |  |
| 81388 | 7315218.644 | 953444.890 | 3282.007 |  |
| 81389 | 7315066.432 | 959011.511 | 3279.391 |  |
| 81390 | 7315097.500 | 964200.986 | 3277.454 |  |
| 81411 | 7309837.749 | 974788.433 | 3234.511 |  |
| 81448 | 7249532.146 | 946701.150 | 3219.903 |  |
| 81475 | 7262219.842 | 962468.457 | 3162.648 |  |
| 81493 | 7272555.832 | 973894.768 | 3188.146 |  |
| 81496 | 7261947.795 | 973328.207 | 3164.077 |  |
| 81499 | 7251383.674 | 974740.589 | 3147.766 |  |
| 81501 | 7246180.145 | 973573.968 | 3145.800 |  |
| 82031 | 7252224.230 | 1000269.002 | 2954.660 |  |
| 83098 | 7232877.178 | 888059.760 | 3318.590 |  |
| 9099 | 7263218.573 | 946682.391 | 3212.771 |  |